DETECDIV

# Pomegranate Pipeline
Installation Guide

MATLAB + dedicated Windows Python environment + CellposeSAM

| Operating system | Windows 10 or Windows 11, 64 bit |
| --- | --- |
| Pipeline | detecdiv_pomegranate, version 1.0 |
| Classifier | CellposeSAM model pombe_seg_1 |
| DetecDiv branch | unstable |
| Installation root | C:\Users\<WindowsUser>\Documents\DetecDivWorkspace |

Follow this guide exactly. Use the folder structure and repository locations shown below. Do not install a separate Python manually and do not reuse a run.json created on another computer.

Validated code references: DetecDiv unstable commit 9d395797; DetecDiv-plugins main commit 29f3b94e.

# 1. Required software

Windows only This procedure uses MATLAB and a local Conda environment on Windows.

| Install manually | Installed automatically by DetecDiv | Download assets |
| --- | --- | --- |
| MATLAB and all 7 MathWorks products | Python 3.10 environment | Pomegranate pipeline ZIP |
| GitHub account and GitHub Desktop | PyTorch 2.7.0, CPU or NVIDIA GPU build | pombe_seg_1 model ZIP |
| Miniconda for Windows | Cellpose 4.2.1.1, Zarr, Trackastra and dependencies | Raw microscopy data |

## Hardware recommendations

16 GB RAM minimum; 32 GB recommended.

At least 20 GB free disk space in addition to MATLAB. The downloadable model ZIP is about 1.5 GB.

An NVIDIA GPU is recommended. DetecDiv automatically falls back to CPU when no compatible NVIDIA GPU is detected.

A stable Internet connection is required for GitHub cloning and the first Python setup.

# 2. Create the required workspace folders

Open File Explorer and create DetecDivWorkspace and its code, assets, data, projects and downloads parent folders. GitHub Desktop will create the two repository folders shown below when cloning.

C:\Users\<WindowsUser>\Documents\DetecDivWorkspace\ ├── code\ │ ├── DetecDiv\ │ └── DetecDiv-plugins\ ├── assets\ │ ├── pipelines\detecdiv_pomegranate\ │ └── models\pombe_seg_1\ ├── data\raw\ ├── projects\ └── downloads

Do not improvise the repository locations. Do not manually create code\DetecDiv or code\DetecDiv-plugins before cloning. GitHub Desktop creates them. Extract the two supplied ZIP files under assets. Store raw images under data\raw and all DetecDiv projects under projects.

# 3. Install MATLAB and the seven required products

Install MATLAB R2021a or newer. During installation, select all seven products below:

☐ MATLAB

☐ Computer Vision Toolbox

☐ Deep Learning Toolbox

☐ Database Toolbox

☐ Image Processing Toolbox

☐ Parallel Computing Toolbox

☐ Statistics and Machine Learning Toolbox

After installation, start MATLAB once and run:

ver

Required means required. Install all seven products even if a specific Pomegranate run does not exercise every feature. This keeps the DetecDiv installation complete and avoids later feature failures.

# 4. Create a GitHub account and install GitHub Desktop

Go to https://github.com/signup in a web browser.

Create a personal GitHub account, verify the email address, and sign in.

Install GitHub Desktop from https://desktop.github.com/.

Open GitHub Desktop and select File > Options > Accounts > Sign in to GitHub.com.

Complete the browser authorization and return to GitHub Desktop.

Recommended account security Enable two-factor authentication in GitHub account settings and keep recovery codes in a safe location.

# 5. Clone DetecDiv with GitHub Desktop

## 5.1 Clone the main repository

In GitHub Desktop, select File > Clone repository.

Open the URL tab.

Repository URL: https://github.com/gcharvin/DetecDiv.git

Local path: C:\Users\<WindowsUser>\Documents\DetecDivWorkspace\code\DetecDiv

Click Clone and wait for the download to finish.

Click Current branch, select unstable, then click Fetch origin and Pull origin if offered.

## 5.2 Clone the plugin repository

Select File > Clone repository again and open the URL tab.

Repository URL: https://github.com/gcharvin/DetecDiv-plugins.git

Local path: C:\Users\<WindowsUser>\Documents\DetecDivWorkspace\code\DetecDiv-plugins

Click Clone. Keep the main branch selected, then click Fetch origin and Pull origin if offered.

## 5.3 Verify both repositories

☐ This file must exist: ...\code\DetecDiv\startup.m

☐ This folder must exist: ...\code\DetecDiv\engine\classification\+cellposesam

☐ This folder must exist: ...\code\DetecDiv-plugins\plugins\processor

# 6. Install Miniconda for Windows

Download the 64-bit Windows Miniconda installer from https://docs.conda.io/projects/miniconda/.

Install for the current user. Keep the default installation directory.

Open Anaconda Prompt and run:

conda --version conda info --base

Close Anaconda Prompt and restart MATLAB after Miniconda installation.

Do not create the DetecDiv environment here. Miniconda is the only Python component installed manually. DetecDiv will create and manage its dedicated environment.

# 7. Initialize DetecDiv in MATLAB

cd('C:\Users\<WindowsUser>\Documents\DetecDivWorkspace\code\DetecDiv') startup which runPipeline which cellposesam.classify

Register and load the external plugin repository:

detecdiv_plugins_register_root( ... 'C:\Users\<WindowsUser>\Documents\DetecDivWorkspace\code\DetecDiv-plugins') detecdiv_plugins_addpath detecdiv_plugins_list

Verify the three Pomegranate processors:

which bestFocusPlane.process which detectViterbiPombeDivisionFrame.process which detecdivPomegranate.process

# 8. Create the dedicated Python environment

Run this once from MATLAB:

info = select_and_load_conda_env( ... 'backend', 'local', ... 'mode', 'default', ... 'debug', true);

## DetecDiv performs all of these actions automatically

Finds Conda and creates the detecdiv_python environment with Python 3.10.

Detects NVIDIA hardware and installs the correct PyTorch 2.7.0 GPU or CPU build.

Installs Cellpose 4.2.1.1, Zarr, Trackastra 0.5.3 and all required Python packages.

Repairs the known Windows OpenMP/NumPy conflict when detected.

Loads the environment into MATLAB with pyenv in OutOfProcess mode.

Runs package and runtime import checks.

## Verify the result

pe = pyenv; disp(pe) disp(info.torch) py.importlib.import_module('torch'); py.importlib.import_module('cellpose.models');

☐ pyenv.Status must be Loaded.

☐ pyenv.ExecutionMode must be OutOfProcess.

☐ Both Python import commands must complete without an error.

# 9. Install the supplied pipeline and model assets

Copy both ZIP files into:

C:\Users\<WindowsUser>\Documents\DetecDivWorkspace\downloads

## 9.1 Extract the pipeline

Right-click detecdiv_pomegranate_pipeline.zip and select Extract All.

Select C:\Users\<WindowsUser>\Documents\DetecDivWorkspace\assets\pipelines as the destination.

Confirm that ...\assets\pipelines\detecdiv_pomegranate\pipeline.json exists.

## 9.2 Extract the CellposeSAM model

Right-click pombe_seg_1_cellposesam_model.zip and select Extract All.

Select C:\Users\<WindowsUser>\Documents\DetecDivWorkspace\assets\models as the destination.

Confirm that ...\assets\models\pombe_seg_1\pombe_seg_1_best.pth exists.

Confirm that ...\assets\models\pombe_seg_1\models\pombe_seg_1 exists.

# 10. Create and run the Pomegranate analysis

Do not use a copied run.json. A run file contains computer-specific absolute paths. Open the supplied pipeline.json and create a new run for this computer.

Start MATLAB and run startup from the DetecDiv repository folder.

Launch DetecDiv by entering detecdiv in the MATLAB Command Window.

Open the pipeline template at ...\assets\pipelines\detecdiv_pomegranate\pipeline.json.

Create a new DetecDiv project under the projects folder.

Select the raw microscopy folder under ...\data\raw.

Create a new pipeline run and choose Local execution.

For classifier_cellposesam_5, select ...\assets\models\pombe_seg_1 as the classifier module folder.

For processor_detecdivpomegranate_7, select an output folder inside the current project folder.

Run a validation or dry-run first, then test one FOV, one ROI and a small frame range.

After the short test succeeds, run the full dataset.

## Expected node outputs

| Pipeline stage | Expected result |
| --- | --- |
| bestFocusPlane | DIC_focus image channel and DIC_focus_best_z dataseries |
| CellposeSAM | results_cellposeSAM_cell instance mask |
| Division tracking | cell_of_interest mask and pombe_division_profile / pombe_division_score |
| Pomegranate export | cell_information, CSV files, Excel workbook and QC PNG images |

# 11. Troubleshooting

| Problem | Required action |
| --- | --- |
| Conda is not found | Open Anaconda Prompt, run conda info --base, restart MATLAB, and rerun select_and_load_conda_env. |
| Wrong Python is already loaded | Close MATLAB completely, reopen it, run startup, and rerun the Python setup command. |
| GPU is not available | Run nvidia-smi in Windows Terminal and update the NVIDIA driver. CPU execution remains available. |
| Processor package is missing | Repeat the plugin registration commands and verify the required packages with which. |
| Model file is missing | Re-extract the model ZIP and link the classifier node to the pombe_seg_1 folder, not directly to the .pth file. |
| A path contains another user name | Create a new run and reselect the raw data, project, model and output folders on this computer. |
| CellposeSAM fails | Collect runner_stdout.txt, runner_stderr.txt and runner_live.log from the run work/cellposesam folder. |

# 12. Final acceptance checklist

☐ All seven MathWorks products are listed by ver.

☐ Both repositories are in DetecDivWorkspace\code and the correct branches are selected.

☐ The three Pomegranate processor packages are found by MATLAB.

☐ detecdiv_python is loaded in OutOfProcess mode.

☐ torch and cellpose.models import successfully.

☐ The pipeline and model archives are extracted under DetecDivWorkspace\assets.

☐ A new local run was created; no foreign run.json was reused.

☐ The short test produced the segmentation mask and Pomegranate output files.

## Information to send when requesting support

MATLAB release and complete ver output.

Full select_and_load_conda_env output with debug=true.

pyenv and info.torch output.

DetecDiv and DetecDiv-plugins branch and commit shown by GitHub Desktop.

The new run.json and the three CellposeSAM runner log files.